# DSP: Filter Design Assignment

# Kalpesh Patil (130040019)			Filter Number: 13

# Filter Specification

## Filter 1

- Nature: Bandpass Filter

- Cutoff Frequencies (in kHz)

s1 =   8700                 	p1 = 10700

p2 =  20700  		s2 =  22700

## Filter 2

- Nature: Bandstop Filter

- Cutoff Frequencies (in kHz)

p1 =     8900                 		s1 = 10900

s2 =     20900       		p2 = 22900

# Filter Design

## Filter 1

### IIR Implementation

Monotonic in Passband as well as Stopband

#### Digital Frequencies

fdigital=fanalog2πfsample

s1  = 0.5466    		p1 = 0.6723

p2 = 1.3006    		s2 = 1.4263

#### Equivalent Analog Frequencies

Ω=tan ω2 ⁡

s1 =   0.2803    		p1 = 0.3494

p2 =  0.7607     		s2 =  0.8650

#### Low Pass Equivalent Filter

Ωl= Ω2-Ω02BΩ

Where
                         Ω02= Ωp1Ωp2    		       and   			B= Ωp2-Ωp1

Stringent of the two values obtained from bandpass specifications is chosen as stop band frequency for low pass filter. Thus specifications of low pass filter needed to be deigned are as follows
Ωlp= 1
Ωls= 1.3561
Monotonic response is needed in both stopband as well as passband, hence Butterworth Filter is designed
N ≥ lnD2D1ln ΩlsΩlp

Considering the minimum order required, we obtain N=8

Poles of this Butterworth Filter are as follows
pk=Ωcei2k+12N+π2

gainDC=ΩcN

Transfer function is given by

Hs= 1.673s8+5.467 s7+14.94 s6+26.5 s5+33.23 s4+30.14 s3+19.33 s2+8.043 s+1.673

Fig1: Poles of Low Pass Equivalent (Butterworth) of IIR BPF

#### Analog Bandpass Filter

To obtain analog bandpass filter, following transformation was carried out in the transfer function
sl= s2+Ω02Bs

After transformation, following transfer function was obtained

Hs  =  0.00137 s8s16+2.248s15+4.654s14+6.027s13+6.96s12+6.14s11+4.834s10+3.079 s9+ 1.753 s8+0.8183s7+0.3415s6+0.1153s5+0.03473s4+0.007995s3+0.001641s2+0.0002107s+2.491*10-5

#### Digital Filter

To obtain analog bandpass filter, following transformation was carried out in the transfer function
s=1-z-11+z-1

After transformation, following transfer function was obtained

Hz= 3.604*10-5 z16- 0.0002883 z14+ 0.001009 z12- 0.002018 z10+ 0.002523 z8- 0.002018 z6+ 0.001009 z4- 0.0002883 z2+ 3.604*10-5z16- 7.301 z15+ 28.06 z14- 73.26 z13+ 143.8 z12- 223.5 z11+ 283 z10- 297.2 z9+ 260.9 z8- 192 z7+ 118.2 z6- 60.22 z5+ 24.98 z4- 8.195 z3+ 2.02 z2- 0.3384 z + 0.03006

Images for the pole-zero plot, magnitude response and phase response obtained using fvtool are shown below

Fig2: Magnitude Response for IIR BPF

fig3: Phase Response for IIR BPF

fig4: Pole-Zero Plot for IIR BPF

#### Direct Form 2

Fig5: Direct form 2 representation of IIR BPF

### FIR Implementation

#### Ideal Impulse Response

hidealn= sinkωc2-sin⁡(kωc1)πn              for n≠0

= ωc2- ωc1π                                      for n= 0

To truncate the ideal response we multiply it by Kaiser window. 	The length of this window and parameter β is determined by using empirical formulae

#### Kaiser Window Parameters

2N+1≥1+ A-82.285ΔωT

where A= -20log10δ         and  ΔωT= ωs- ωp
By substituting values we get, N=15, A=18.0158, α=0 and β=0 
We take N=20 to match specifications

#### Filter Response

Filter Coefficients [41] = [0.0193    0.0257    0.0078   -0.0023    0.0099    0.0153   -0.0123   -0.0465   - 0.0389    0.0070    0.0339    0.0150   -0.0004    0.0356    0.0764    0.0264   -0.1102   -0.1888   -0.0853    0.1293    0.2400    0.1293   -0.0853   -0.1888   -0.1102    0.0264    0.0764    0.0356   -0.0004    0.0150    0.0339    0.0070   -0.0389   -0.0465   -0.0123    0.0153    0.0099   -0.0023    0.0078    0.0257    0.0193]

Fig6:Magnitude Response of FIR BPF

fig7:Phase Response for FIR BPF

fig 8:Impulse Response for FIR BPF

fig9:Zeros of FIR BPF

## Filter 2

### IIR Implementation

Equiripple in Passband and Monotonic Stopband

#### Digital Frequencies

fdigital=fanalog2πfsample

p1 = 0.5592    		s1 = 0.6849

s2 = 1.3132    		p2 = 1.4388

#### Equivalent Analog Frequencies

Ω=tan ω2 ⁡

p1 =  0.2871    	              s1 =   0.3565

s2 =   0.7707    		p2 =  0.8761

#### Low Pass Equivalent Filter

Ωl= BΩΩ02-Ω2

Where 
                        Ω02= Ωp1Ωp2    		              and   			B= Ωp2-Ωp1

Stringent of the two values obtained from bandpass specifications is chosen as stop band frequency for low pass filter. Thus specifications of low pass filter needed to be deigned are as follows
Ωlp= 1
Ωls= 1.3256
Equiripple response is needed in passband and monotonic in stopband, hence Chebyshev Filter is designed
N ≥ cosh-1D2D1cosh-1ΩlsΩlp

Considering the minimum order required, we obtain N=4

Ak=2k+1π2N
Bk= sinh-11ϵN

Poles are given by
pk= ΩpsinAksinhBk+iΩpcosAkcoshBk

Transfer function is given by
 Hs= 0.2017s4+ 0.8342 s3+ 1.348 s2+ 0.6243 s + 0.2373

Fig10: Poles of Low Pass Equivalent of IIR BSF (Chebyshev)

#### Analog Bandpass Filter

To obtain analog bandstop filter, following transformation was carried out in the transfer function

sl= Bss2+Ω02

After transformation, following transfer function was obtained

Hs=   0.85 s8+ 0.8552 s6+ 0.3227 s4+ 0.05411 s2+ 0.003403 s8+1.549 s7+2.976 s6+1.887 s5+1.878 s4+0.4747 s3+0.1883 s2+0.02466 s +0.004003

#### Digital Filter

To obtain analog bandpass filter, following transformation was carried out in the transfer function

s=1-z-11+z-1

After transformation, following transfer function was obtained

Hz=  0.2089 z8- 0.9995 z7+ 2.629 z6- 4.428 z5+ 5.267 z4- 4.428 z3 + 2.629 z2- 0.9995 z + 0.2089z8- 3.115 z7+ 5.066 z6- 5.76 z5+ 4.999 z4- 3.181 z3+ 1.598 z2- 0.716 z + 0.2114

Images for the pole-zero plot, magnitude response and phase response obtained using fvtool are shown below

fig11: Magnitude Response of IIR BSF

fig12: Phase Response of IIR BSF

fig13: Pole-Zero Plot of IIR BSF

#### Direct Form 2

fig14:  Direct Form 2 
	Representation of 
	IIR BSF

### FIR Implementation

#### Ideal Impulse Response

hidealn= sinkωc1-sin⁡(kωc2)πn              for n≠0

= 1-ωc2- ωc1π                              for n= 0

To truncate the ideal response we multiply it by Kaiser window. 	The length of this window and parameter β is determined by using empirical formulae

#### Kaiser Window Parameters

2N+1≥1+ A-82.285ΔωT

where A= -20log10δ         and  ΔωT= ωs- ωp

By substituting values we get, N=18, A=18.0158, α=0 and β=0

#### Filter Response

Filter Coefficients[41]  =

[-0.0129   -0.0256   -0.0110    0.0014   -0.0095   -0.0187    0.0058    0.0439    0.0436   -0.0003   -0.0316   -0.0160    0.0014   -0.0332   -0.0784   -0.0338    0.1043    0.1900    0.0903   -0.1268    0.7600   -0.1268    0.0903    0.1900    0.1043   -0.0338   -0.0784   -0.0332    0.0014   -0.0160   -0.0316   -0.0003    0.0436    0.0439    0.0058   -0.0187   -0.0095    0.0014   -0.0110   -0.0256   -0.0129]

Fig15: Magnitude Response of FIR BSF

Fig16: Phase Response of FIR BSF

fig17: Impulse Response of FIR BSF

Fig18: Zeros of FIR BSF

# Codes

### IIR Bandpass Filter (Using Butterworth)

% % Kalpesh Patil - 130040019

% % filter number 13

% % BPF IIR

% % Butterworth

%finding cutoff frequencies

m                       =   13;

q_m                     =   floor(0.1*m);

r_m                     =   m - 10*q_m;

Bl_m                    =   4 + 0.7*q_m + 2*r_m;

Bh_m                    =   Bl_m + 10;

delta_1                 =   0.15;

delta_2                 =   0.15;

f_sample                =   100*1000;

omega_s1                =   (Bl_m-2)*1000;

omega_p1                =   (Bl_m)*1000;

omega_p2                =   (Bh_m)*1000;

omega_s2                =   (Bh_m+2)*1000;

f_analog_array          =   [omega_s1 omega_p1 omega_p2 omega_s2];

%normalized specs

f_digital_array         =   f_analog_array.*2*pi/f_sample;

f_eqv_analog            =   tan(f_digital_array/2);

%analog LPF

omega_not               =   sqrt(f_eqv_analog(2)*f_eqv_analog(3));

B                       =   f_eqv_analog(3) - f_eqv_analog(2);

%frequency transformation

f_eqv_analog_LPF        =   (f_eqv_analog.^2 - omega_not^2)./(B*f_eqv_analog);

%designing this LPF

D1  				=   (1/(1-delta_1)^2)-1;

D2  				=   (1/delta_2^2)-1;

mod_f_eqv_analog_LPF    =   abs(f_eqv_analog_LPF);

stringent_omega_s       =   min(mod_f_eqv_analog_LPF(1),mod_f_eqv_analog_LPF(4));

temp                    =   log(sqrt(D2)/sqrt(D1))/log(stringent_omega_s/f_eqv_analog_LPF(3));

N                       =   ceil(temp);

%finding poles of equivalent low pass filter

omega_p                 =   f_eqv_analog_LPF(3);

omega_c                 =   ((omega_p/(D1^(1/(2*N))))+(stringent_omega_s/(D2^(1/(2*N)))))/2;

thetas                  =   (2*[1:2*N] - 1)*(pi)/(2*N);

gain_LF                 =   omega_c^N;

poles                   =   1i*omega_c*exp(1i*thetas);

poles_LF                =   poles(1:N);

zeros_LF                =   [];

[num_LF,den_LF]         =   zp2tf(zeros_LF,poles_LF,gain_LF);

tf_LF                   =   tf(num_LF,den_LF);

%converting back to band pass filter

poles_BF                =   zeros(1,2*N);

poles_BF(1:N)           =   (B/2).*(poles_LF-sqrt(poles_LF.^2 - 4*omega_not^2/(B^2)));

poles_BF(N+1:2*N)       =   (B/2).*(poles_LF+ sqrt(poles_LF.^2 - 4*omega_not^2/(B^2)));

zeros_BF                =   zeros(1,N);

gain_BF                 =   gain_LF*B^N;

[num_BF,den_BF]         =   zp2tf(zeros_BF',poles_BF,gain_BF);

tf_BF                   =   tf(num_BF, den_BF);

%converting to z domain

poles_BF_z              =   (1 + poles_BF)./(1 - poles_BF);

temp                    =   prod(1-poles_BF);

gain_BF_z               =   gain_BF/temp;

zeros_BF_z              =   [ones(1,N),-ones(1,N)];

[num_BF_z, den_BF_z]    =   zp2tf(zeros_BF_z',poles_BF_z,gain_BF_z);

DigitalFilter           =   tf(num_BF_z, den_BF_z,1/f_sample);

h                       =   fvtool(num_BF_z, den_BF_z);

### FIR Bandpass Filter

% % Kalpesh Patil - 130040019

% % filter number 13

% % BPF FIR

m                      = 13;

q_m                    = floor(0.1*m);

r_m                    = m - 10*q_m;

Bl_m                   = 4 + 0.7*q_m + 2*r_m;

Bh_m                   = Bl_m + 10;

delta_1                = 0.15;

delta_2                = 0.15;

f_sample               = 100*1000;

omega_s1               = (Bl_m-2)*1000;

omega_p1               = (Bl_m)*1000;

omega_p2               = (Bh_m)*1000;

omega_s2               = (Bh_m+2)*1000;

f_analog_array         = [omega_s1 omega_p1 omega_p2 omega_s2];

%normalized specs

f_digital_array        = f_analog_array.*2*pi/f_sample;

% Kaiser window parameters

del_omega1             = f_digital_array(4)-f_digital_array(3);

del_omega2             = f_digital_array(2)-f_digital_array(1);

del_omega              = min(abs(del_omega1),abs(del_omega2));

A                      = -20*log10(del_omega);

% Order

N_1                    =  (A-8)/(2*2.285*del_omega);

N_limit                =  ceil(N_1);

N                      =  N_limit + 5;

if(A<21)

alpha=0;

else if(A<=50)

alpha          =  0.5842*(A-21)^0.4+0.07886*(A-21);

else

alpha          =  0.1102*(A-8.7);

end

end

omega_c1               =  (f_digital_array(2)+f_digital_array(1))*0.5;

omega_c2               =  (f_digital_array(4)+f_digital_array(3))*0.5;

h_ideal = [];

for k = -N:N

if(k~=0)

h_ideal(k+N+1) = (sin(omega_c2*k)-sin(omega_c1*k))/(pi*k);

else

h_ideal(k+N+1) =  0;

end

end

h_ideal(N+1)           =  ((omega_c2-omega_c1)/pi);

beta                   =  alpha/N;

% Generating Kaiser window

h_kaiser               =  kaiser(2*N+1,beta);

h_org                  =  h_ideal.*h_kaiser';

fvtool(h_org);

### IIR Bandstop Filter (Using Chebyshev)

% % Kalpesh Patil - 130040019

% % filter number 13

% % BSF IIR

% % Chebyshev

%finding cutoff frequencies

m                       =   13;

q_m                     =   floor(0.1*m);

r_m                     =   m - 10*q_m;

Bl_m                    =   4 + 0.9*q_m + 2*r_m;

Bh_m                    =   Bl_m + 10;

delta_1                 =   0.15;

delta_2                 =   0.15;

f_sample                =   100*1000;

omega_p1                =   (Bl_m-2)*1000;

omega_s1                =   (Bl_m)*1000;

omega_s2                =   (Bh_m)*1000;

omega_p2                =   (Bh_m+2)*1000;

f_analog_array          =   [omega_p1 omega_s1 omega_s2 omega_p2];

%normalized specs

f_digital_array         =   f_analog_array.*2*pi/f_sample;

f_eqv_analog            =   tan(f_digital_array/2);

%analog LPF

omega_not               =   sqrt(f_eqv_analog(1)*f_eqv_analog(4));

B                       =   f_eqv_analog(4) - f_eqv_analog(1);

%frequency transformation

f_eqv_analog_LPF        =   (B*f_eqv_analog)./(omega_not^2 - f_eqv_analog.^2);

%designing this LPF

D1                      =   (1/(1-delta_1)^2)-1;

D2                      =   (1/delta_2^2)-1;

epsilon                 =   sqrt(D1);

mod_f_eqv_analog_LPF    =   abs(f_eqv_analog_LPF);

stringent_omega_s       =   min(mod_f_eqv_analog_LPF(2),mod_f_eqv_analog_LPF(3));

temp                    =   acosh(sqrt(D2)/sqrt(D1))/acosh(stringent_omega_s/f_eqv_analog_LPF(1));

N                       =   ceil(temp);

%finding poles of equivalent low pass filter

omega_p                 =   mod_f_eqv_analog_LPF(4);

Ak                      =   ([1:2*N]*2 + 1)*pi/(2*N);

Bk                      =   asinh(1/epsilon)/N;

temp_real               =   omega_p*sin(Ak)*sinh(Bk);

temp_imag               =   omega_p*cos(Ak)*cosh(Bk);

temp                    =   temp_real + 1i*temp_imag;

poles_LF                =   zeros(1,N);

t                       =   1;

for k = 1:2*N

if(temp_real(k) < 0)

poles_LF(t)       =   temp(k);

t                 =   t+1;

end

end

if (mod(N,2) == 0)

d                   =   1/sqrt(1+epsilon^2);

else

d                   =   1;

end

g                       =   ((-1)^N)*prod(poles_LF);

gain_k                  =   d*g;

zeros_LF                =   [];

[num_LF,den_LF]         =   zp2tf(zeros_LF',poles_LF,gain_k);

tf_LF                   =   tf(num_LF,den_LF);

%converting back to band stop filter

poles_BF                =   zeros(1,2*N);

poles_BF(1:N)           =   (B/2).*(1./poles_LF + sqrt(1./poles_LF.^2 - 4*omega_not^2/(B^2)));

poles_BF(N+1:2*N)       =   (B/2).*(1./poles_LF - sqrt(1./poles_LF.^2 - 4*omega_not^2/(B^2)));

zeros_BF                =   [1i*omega_not*ones(1,N)' ; -1i*omega_not*ones(1,N)'];

gain_BF                 =   gain_k/g;

[num_BF,den_BF]         =   zp2tf(zeros_BF,poles_BF,gain_BF);

tf_BF                   =   tf(num_BF, den_BF);

%converting to z domain

poles_BF_z              =   (1 + poles_BF)./(1 - poles_BF);

temp                    =   prod(1-poles_BF);

gain_BF_z               =   gain_BF*((1+omega_not^2)^N)/(prod(1-poles_BF));

zeros_BF_z              =   (1 + zeros_BF)./(1 - zeros_BF);

[num_BF_z, den_BF_z]    =   zp2tf(zeros_BF_z,poles_BF_z,gain_BF_z);

DigitalFilter           =   tf(num_BF_z, den_BF_z,1/f_sample);

h                       =   fvtool(num_BF_z, den_BF_z);

### FIR Bandstop Filter

% % Kalpesh Patil - 130040019

% % filter number 13

% % BSF FIR

m                      = 13;

q_m                    = floor(0.1*m);

r_m                    = m - 10*q_m;

Bl_m                   = 4 + 0.9*q_m + 2*r_m;

Bh_m                   = Bl_m + 10;

delta_1                = 0.15;

delta_2                = 0.15;

f_sample               = 100*1000;

omega_p1               = (Bl_m-2)*1000;

omega_s1               = (Bl_m)*1000;

omega_s2               = (Bh_m)*1000;

omega_p2               = (Bh_m+2)*1000;

f_analog_array         = [omega_p1 omega_s1 omega_s2 omega_p2];

%normalized specs

f_digital_array        = f_analog_array.*2*pi/f_sample;

% Kaiser window parameters

del_omega1             = f_digital_array(4)-f_digital_array(3);

del_omega2             = f_digital_array(2)-f_digital_array(1);

del_omega              = min(abs(del_omega1),abs(del_omega2));

A                      = -20*log10(del_omega);

% Order

N_1                    =  (A-8)/(2*2.285*del_omega);

N_limit                =  ceil(N_1);

N                      =  N_limit + 5;

if(A<21)

alpha=0;

else if(A<=50)

alpha          =  0.5842*(A-21)^0.4+0.07886*(A-21);

else

alpha          =  0.1102*(A-8.7);

end

end

omega_c1               =  (f_digital_array(2)+f_digital_array(1))*0.5;

omega_c2               =  (f_digital_array(4)+f_digital_array(3))*0.5;

h_ideal = [];

for k = -N:N

if(k~=0)

h_ideal(k+N+1) = - (sin(omega_c2*k)-sin(omega_c1*k))/(pi*k);

else

h_ideal(k+N+1) =   0;

end

end

h_ideal(N+1)           =  1 - ((omega_c2-omega_c1)/pi);

beta                   =  alpha/N;

% Generating Kaiser window

h_kaiser               =  kaiser(2*N+1,beta);

h_org                  =  h_ideal.*h_kaiser';

fvtool(h_org);